| Company Name/logo | SSOP – Chilling and Packaging Room Example | Version No.: 01 |
| --- | --- | --- |
| Effective Date: mm-dd-yyyy | Approved by: [Insert Manager/ Supervisor responsible] | Revision Date: mm-dd-yyyy |

| Chemicals Used | Equipment Used |
| --- | --- |
| - Foamforce: 500-1000 ppm free chlorine - ZEP PDC: 200-400ppm Refer to ‘Approved Chemical List’ for mixing instructions | - High-pressure washer (800 psi minimum, water temperature 130°F) - Foamer gun (e.g. Gilmour Chemical Foamer) - Metal handle squeegees (floor use only) - Long-handle floor brooms and dustpans (floor use only) - Small blue short-handle brushes (food contact surfaces only) - Inedible waste tubs and edible shovels - Sanitizer hose |

NO CLEANING IN CHILLING AND PACKAGING ROOM UNTIL ALL PRODUCTS HAVE BEEN PACKAGED AND REMOVED TO THE FREEZER OR COOLER

### Purpose:

To ensure effective cleaning and sanitation of chilling and packaging room.

### Who:

Trained Workers

### When:

- After the last bird is packaged and removed to the cooler or freezer.

- Every processing day.

### Procedure:

- Turn on the exhaust fan.

- Drain the chill tanks.

- Empty the ice machine.

- Ensure all packaging and materials are removed from room other than cleaning materials.

- Using gloved hands or long handle floor brooms & dustpans, remove all large debris, scraps, and materials from the floor and place into an inedible bin. Do not wash debris down the drain.

- Using the red hose (130°F water), rinse down the walls, lights, chill tanks, icebox, racks, equipment and floor. Wash all scraps toward the drain.

- Apply foaming detergent- Foamforce@500-1000ppm to all equipment, tanks, containers, walls, floors, drip-racks, doors, icebox, sinks, lights and drains.

- After 10 minutes contact time, handwash the icebox and scales with clean short, handled brushes. High-pressure wash all walls, shackles, doors (both sides), sink, equipment and floor. Remove floor drain cover and high-pressure wash the grate, gutter and drain opening. Replace the floor drain cover.

- With the red hose rinse all walls, lights, doors and equipment and floor toward the drain.

- Using the squeegee and shovel, pick up any remaining small scraps. Open the floor drain; complete rinsing of floor, sewer grate and drain. Flush drain trap of all scraps.

- Let the room dry for two hours.

- Apply the sanitizer- ZEP-PDS@200-400ppm to all washed surfaces, equipment, floor and drain.

- Turn off exhaust fan when room and equipment are dried.

### Deviation and Corrective Action Procedures:

If the area and/or equipment is found unclean during a pre-operational inspection by the monitor, it is considered a deviation, and the area and/or equipment must be re-cleaned before usage according to the SSOP procedure. Then, record the deviation and corrective action in the Pre-op Inspection Record.

### Verification Procedures:

For any deviation, the corrective action is verified by the operator or a designated verifier to ensure that the area and/or equipment is clean before usage, as a part of follow-up for non-conformance. The verifier and the monitor cannot be the same person.

For verification of the cleaning procedures being followed according to the written program, the verifier will do an on-site verification semi-annually.

### Records:

- Pre-operational Inspection Record

- Sanitation Schedule- Chilling and Packaging Room

### Review:

This procedure needs to be reviewed annually and revised in case of any changes in the procedures. Any revision or review needs to be recorded in the Revision/Review Log, with the date of revision/review, version no., description of change and the personnel involved in revision.

8.1 Revision/ Review Log:

| Version No. | Revision Date/ Annual review date | Description of Change | Revised/ Reviewed by |
| --- | --- | --- | --- |