| Company Name/logo | SSOP- Cooler- Example | Version No.: 01 |
| --- | --- | --- |
| Effective Date: mm-dd-yyyy | Approved by: [Insert Manager/ Supervisor responsible] | Revision Date: mm-dd-yyyy |

| Chemicals Used | Equipment Used |
| --- | --- |
| - Foamforce: 500-1000 ppm free chlorine - ZEP PDC: 200-400ppm Refer to ‘Approved Chemical List’ for mixing instructions | - High-pressure washer (800 psi minimum, water temperature 130°F) - Foamer gun (e.g. Gilmour Chemical Foamer) - Metal handle squeegees (floor use only) - Long-handle floor brooms (floor use only) - Small blue short-handle brushes (food contact surfaces only) - Sanitizer hose |

NO CLEANING OF THE COOLER UNTIL THE UNIT IS EMPTY

- Purpose:

To ensure effective cleaning and sanitation of the cooler.

- Who:

Trained Workers

- Frequency:

Immediately after products are shipped and/or the cooler is empty.

# 4. Procedure:

  - Sanitation Procedures

- Prepare the room

- Remove all products to the freezer or to the secondary cooler. Turn off the refrigeration unit.

- Remove portable racks, shelving units, and bins to a clean area for separate cleaning.

- Sweep and shovel all loose debris from the floor into waste containers. Do not hose solids down the drain.

- Remove floor drain covers and screens and place them on the floor for separate cleaning.

- Inspect the refrigeration/condenser unit and remove any visible debris or build up and place it into a waste container.

- Pre-rinse

- Using the red hose with hot potable water (130°F / 54°C), rinse walls, lights, racks, and floor from top to bottom.

- Wash all scraps toward the drain; pick up with the floor shovel. Do not spray scraps down the drain.

- Apply Detergent

  - Apply foam cleaner Foamforce @ 500–1000 ppm to the walls, floors, shelves, doors (both sides), lights, and drains. Work from bottom to top.

  - Apply Foamforce @ 500–1000 ppm to the refrigeration/condenser unit. The cleaner will be rinsed off within the 10-minute contact time to prevent damage to the unit.

  - Allow foam to sit on all other surfaces for the contact time of 5–10 minutes.

- Rinse

  - Remove the floor drain cover and high-pressure wash the grate and drain opening. Replace the floor drain cover.

  - Using the red hose with hot potable water (130°F / 54°C), rinse all walls, lights, doors, and floor toward the drain.

  - Using the squeegee and shovel, pick up remaining small scraps. Open the floor drain; complete rinsing of the floor, sewer grate, and drain. Flush the drain trap of all scraps.

- Inspect

- Inspect all surfaces and equipment using sight, touch, or smell to verify they are free from visible soil, residue, odour, and detergent film.

- Pay particular attention to rack and shelf joints, door gaskets and seals, wall–floor junctions, drain covers, and the refrigeration unit coils and drip pan.

- Do not sanitize unless surfaces are visibly clean.

- Sanitize

- Leave the door open until the room is dry.

- Apply sanitizer ZEP PDC @ 200–400 ppm as a no-rinse sanitizer to all washed surfaces, shelving, floor, and drain. Allow to air dry.

- Apply ZEP PDC @ 200–400 ppm to the refrigeration unit. Allow a five-minute contact time and rinse off with the sanitizer hose.

- Return racks and shelving to their designated positions once dry.

- If product is to be returned to the cooler, turn on the refrigeration unit and monitor the temperature to ensure adequate operation before returning product (0°C to 4°C). If no product is to be returned, leave the door open to air dry and keep the refrigeration unit turned off.

- Complete the 'Sanitation Schedule – Cooler' by recording the date, time, worker initials, deviations (if any), and corrective actions (if any).

  - Operational Sanitation Procedures

- Condensation or Pooling Water

- Monitor the floor for pooled water, remove any pooled water and clear any drain blockages immediately.

- Monitor the refrigeration/condenser unit for excessive condensation or ice build-up on coils, which may indicate a malfunction.

- Visible Contamination

- Identify and immediately address any visible contamination on racks, shelving, or walls. Notify the operator if product contamination is suspected; segregate and manage per the facility's contamination control SOP.

5. Deviation and Corrective Actions:

During pre-operational inspection, if the area or equipment surface does not pass visual inspection, it will be re-cleaned, re-inspected and sanitized according to SSOP. The area will not be sanitized until it passes visual inspection. This will be recorded in the deviation and corrective action section of ‘Pre-operational Inspection Record’ with the initials of the monitor.

6. Verification:

Follow up of efficacy of corrective action in performed as a part of verification procedure during record review.

Verification of written SSOP is performed on-site by the operator or a designate at least once a year for all areas and equipment. The verifier observes the cleaning procedures on-site to ensure it follows the written procedures.

# 7. Records:

- Pre-operational Inspection Record

- Sanitation Schedule- Cooler

- Operational Sanitation Log

8. Review:

This procedure needs to be reviewed annually and revised in case of any changes in the procedures. Any revision or review needs to be recorded in the Revision/Review Log, with the date of revision/review, version no., description of change and the personnel involved in revision.

8.1 Revision/ Review Log:

| Version No. | Revision Date/ Annual review date | Description of Change | Revised/ Reviewed by |
| --- | --- | --- | --- |