Section Cover Page

Section 32 92 23 2026-04-07 Sodding

Use this Section to specify requirements for sodding in conjunction with Section 32 91 19 - Topsoil Placement and Grading.

This Master Specification Section contains:

.1	This Cover Sheet

.2	Specification Section Text:

# 1.	General

1.1	Related Requirements

1.2	Reference Documents

1.3	Submittals

1.4	Quality Assurance

1.5	Measurement And Payment

1.6	Examination

1.7	Delivery, Storage And Handling

1.8	Maintenance Period

1.9	Warranty

# 2.	Products

2.1	Material

2.2	Sod Type

# 3.	Execution

3.1	Weather Limitations

3.2	Surface Preparation Of Sodbed

3.3	Fertilizer – Initial Application

3.4	Sod Placement

3.5	Laying Sod On Slopes And Drainage Channels

3.6	Protection Of Sodded Areas

3.7	Maintenance And Plant Establishment

3.8	Maintenance Log Requirements

3.9	Final Acceptance And Termination Of Maintenance

3.10	Clean-Up And Repairs

# 1.	General

# 1.1		RELATED REQUIREMENTS

.1	Topsoil Placement and Grading:	Section 32 91 19.

.2	Exterior Landscape Maintenance:	Section 32 99 00.

# 1.2	REFERENCE DOCUMENTS

.1	Canadian Standards for Nursery Stock - (Nursery Sod):  latest edition by the Canadian Nursery Landscape Association (CNLA).

.2	“Environmental Protection and Enhancement Act” – Alberta Environment regulations

regarding pesticides.

1.3	SUBMITTALS

SPEC NOTE:  edit submittal requests to suit contract requirements.

.1	Comply with Submittal Procedures Section 01 33 00:

.1	Sod growers certification of grass species and location of sod source.

.2	Copies of all permits and licenses as applicable to work of this contract.

.3	Work schedule showing approximate dates for commencement and completion of sodding activities and other related landscape work.

.4	Site supervisor’s credentials prior to commencement of work.

.5	Pesticide applicator’s license in control of any perennial weeds and grasses on site.  Submit only if pesticide application services are applicable.

# 1.4	quality assurance

.1	Complete work under the on-site direction and supervision of a certified landscape professional.  Site supervisor shall possess a Journeyman Landscape Horticulturist certification or have other equivalent qualifications acceptable to the Province.

.2	Contractor shall be responsible for ensuring that contract specifications are being adhered to.  Failure of the Province to immediately reject unsatisfactory workmanship, materials or to notify the Contractor of their deviation from the specification shall not relieve the Contractor of their responsibility to repair and/or replace unsatisfactory work.

.3	Sod: turfgrass sod shall be of superior quality that meets the standards and requirements of the “Canadian Standards for Nursery Stock – (Nursery Sod)”.

.4	Pre-Installation Meetings:  prior to commencement of work, Contractor shall conduct pre-installation meeting with the Province to verify project requirements, installation instructions and maintenance/warranty requirements.

# 1.5	measurement and payment

SPEC NOTE: ensure measurement and payment conditions match other respective landscape sections in contract.   Delete reference to either stipulated price contracts or unit price contracts as applicable to contract.

.1	Stipulated Price (Lump Sum) Contracts:

.1	For payment purposes, landscape work of contract in contract shall be paid in accordance with the following guidelines.  The Province reserves the right to adjust payment conditions where deemed necessary.

.1	Seventy-five (75%), of the total landscape contract value shall be deemed to be allocated for supply and installation of landscape materials and related work specified in contract documents.

.2	Remaining twenty-five (25%), of the total landscape contract value shall be deemed to be allocated for landscape maintenance and warranty services specified in contract documents.

.2	Unit Price Contracts:

.1	Unit price items of work shall be measured in-place and paid at the contract unit bid price.  Unit bid price shall include full compensation for all materials, equipment, labour and incidentals necessary to complete each unit item of work.

1.6	examination

.1	Contractor shall verify all information on drawings with actual field conditions and inspect related work and adjacent surfaces.  Contractor shall advise the Province of any conditions or defects encountered that prevent proper execution of the work.

.2	Do not commence work until these conditions or defects have been evaluated and corrective measures taken.

.3	Commencement of work shall imply acceptance of existing surfaces and conditions.  No  claims for damages or extras resulting from such conditions or defects will be accepted thereafter, except in cases where such conditions cannot be known prior to or during the course of construction.

1.7	DELIVERY, STORAGE AND HANDLING

.1	Use all means necessary to protect all materials before, during and after installation and to protect the installed work and materials of other trades.

.2	Deliver fertilizer to site in original, unopened and non-damaged water-proof packages, each bearing manufacturer’s guaranteed analysis.

.3	Schedule deliveries to minimize site storage and sod placement delays.

.4	Protect turfgrass sod from sun, wind, dehydration, and rain during delivery and prior to installation.   Keep sod moist, cool and protected against adverse weather conditions until installed.

.5	Time Limitations: sod shall be harvested, delivered and installed within a period of 36 hours.  Sod not installed within 36 hours of harvesting or within 24 hours of delivery shall be rejected if there is any evidence of deterioration.

.6	Sod showing visible signs of heating or dehydration shall be rejected.  Dried out, damaged, deteriorated or unhealthy sod shall be rejected.  Broken or irregular pieces of sod shall be rejected.  Immediately remove all unacceptable sod from site.

1.8	MAINTENANCE PERIOD

SPEC NOTE: edit item to indicate required length of maintenance period in contract.  Ensure that the specified period matches the maintenance period specified in other respective landscape sections.

.1	Maintain sod and all other landscape work in contract from time of installation until minimum [one (1) year] [two (2) years] after Date of Interim Acceptance of the landscape portion of Work in Contract.

.2	Maintenance services shall generally be performed during the landscape growing season from May 1st to October 31st, weather permitting.  The Province, however, reserves the right to have maintenance services commence at an earlier date and terminate at a latter date based on existing climatic conditions, soil moisture and plant health.

.3	Incomplete weeks or months of maintenance shall be carried over to the following landscape growing season (as applicable).

.4	The Province reserves the right to extend maintenance period and/or reduce progress payments for maintenance services any time Contractor neglects to provide proper and adequate maintenance services in accordance with contract specifications as determined by the Province.

.5	Any additional maintenance beyond the specified maintenance period required for Contractor to achieve final acceptance of work shall be provided at Contractor’s own expense.

1.9	warranty

SPEC NOTE: edit item to indicate the length of warranty period in contract.  Landscape warranty periods shall match the length of the maintenance period in contract.

.1	Provide warranty for all sod and other related work and materials for a minimum period of [one (1) year] [two (2) years].  Warranty period shall commence from date of Interim Acceptance of the landscape portion of Work in Contract.

.2	The Province reserves the right to extend Contractor’s warranty responsibilities for an additional period of time, if at the end of the initial warranty period, sod development, colour and growth are not sufficient to ensure future survival.  Length of any additional warranty period shall be determined by the Province.

.3	During warranty period, Contractor shall continuously replace all sod which is dead, unhealthy, deficient or in unsatisfactory growing condition, and as often as required to establish the sodded lawn area until accepted.  Install replacement sod in accordance with contract specifications at no additional compensation.

.4	Replacement Sod Warranty: the Province reserves the right to extend Contractor’s warranty obligations on replacement sod work that is not fully established.  Length of any additional warranty period will be determined by the Province.  Additional warranty periods shall be provided at no additional cost to the Province.

.5	The Province shall be sole judge as to condition of sod regarding warranty replacements.

# 2.	Products

# 2.1	MATERIAL

SPEC NOTE: edit item to suit contract requirements. Delete all materials that are not applicable.

.1	Fertilizer:  to Canada “Fertilizers Act” and “Fertilizers Regulations”.

.1	Sod Starter Fertilizer: use standard synthetic granular commercial fertilizers formulated with a higher phosphorus content to promote root development on new turfgrass sod installations.

.2	Established Turfgrass Fertilizer: use a complete synthetic granular turf fertilizer containing minimum 50% of total nitrogen derived from a slow or controlled release nitrogen source to promote healthy growth during maintenance period.

.2	Nursery Sod:  certified number one turfgrass sod of premium grade.  Sod shall comply with standards outlined in current edition of “Canadian Standards for Nursery Stock - (Nursery Sod)”, published by the Canadian Nursery Landscape Association.

.1	Permeated with a strong fibrous root system; be freshly cut and in good healthy condition with no decay or bare spots; be uniform in texture and free from weeds, undesirable native grasses and foreign debris; and contain adequate moisture to maintain its vitality during transportation and placement.

.2	Grown on fertile topsoil, by a sod producer specializing in sod production and harvesting.  Sod shall be 18 to 24 months in age prior to harvesting.  Sod grown in peat bedded soil is not acceptable.

.3	Broken, dry, deteriorated, damaged, discoloured and thin sod is not acceptable.

.4	Machine cut at a uniform width and length and with a minimum soil thickness of 15mm, excluding top growth.  Each individual sod piece shall be strong enough to support its own weight when lifted by the ends.

.5	Remove all netting or mesh on big sod rolls prior to laying sod where applicable.

.3	Water:  clean, fresh, and free of substances or matter that would inhibit vigorous and healthy turf growth.

.1	Contractor shall be responsible for the supply of clean water, water truck, pumps, hoses, and other related equipment and accessories for all sodding requirements, sod maintenance and other related work.

.2	All costs for supply of water incurred during the contract period shall be borne by Contractor.

.3	Where available, the Province shall provide a limited supply of water from building source at no cost to Contractor.

.4	Staples:  u-shaped steel wire staples, 150 mm to 200 mm in length as recommended by manufacturer of geotextile fabric material.

.5	Geotextile Fabric Mesh:  biodegradable 25 mm square netting for slope reinforcement.

.6	Biodegradable Pegs:  17 mm x 8 mm x 200 mm.

.7	Wooden Pegs: 19 mm x 19 mm x 150 to 200 mm long.

# 2.2		SOD TYPE

SPEC NOTE: edit item and indicate type of sod(s) required.  Sod shall be selected to suit site conditions, site use and maintenance requirements.

.1	Premium Kentucky Bluegrass/Fine Fescue Sod:  nursery sod grown from certified Canada No. 1 seed mix containing not less than a 90% blend of 3 to 4 Kentucky bluegrass cultivars and the remainder consisting of a blend of Fine Fescue cultivars.

.2	Drought tolerant Kentucky Bluegrass/Fine Fescue Sod:  nursery sod grown from certified Canada No. 1 seed mix containing approximately a 40% blend of Kentucky bluegrass cultivars and approximately a 60% blend of Fine Fescue cultivars.

.3	Fescue Sod:  specialty drought and shade tolerant nursery sod grown from certified Canada No. 1 fescue grass seeds.  Seed mixture shall be composed of a 100% mixture of Creeping red fescue, Chewing fescue, Hard fescue and Sheep fescue grass seeds.

# 3.	Execution

# 3.1	weather limitations

.1	Sodding shall be performed during favourable weather conditions and only during normal and acceptable planting seasons when satisfactory growing conditions exist.

.2	Do not lay sod during periods of high summer temperatures or during periods of drought when a consistent supplementary supply of water is not available to retain sod and underlying soil in moist condition.  Do not lay sod during periods of heavy rainfall and when puddling is apparent on site.  Never lay sod when ground is frozen or covered in snow or during other times of unfavourable climatic conditions.

.3	Sod shall not be installed after October 15th unless weather and soil conditions are favourable for sod establishment and the Province allows the work to proceed.

.4	The Province, however, reserves the right to not accept any sod installed after October 15th until minimum thirty (30) days after start of the following spring growing season as determined by the Province provided work was performed in accordance with the specifications.

# 3.2	surface preparation of sodbed

.1	Prior to sodding, remove any existing stones larger than 25 mm in diameter, weeds, grass, roots, clods, foreign debris and other extraneous materials from sodbed and dispose off-site.  Remove any contaminated soil and replace with acceptable fertile topsoil.

.2	Lightly rake, loosen and shape sodbed soil to a fine granular texture prior to sodding.  Ensure all uneven grades, erosion, settlement or disturbances that have resulted since topsoil was placed are eliminated.

.3	Where new sod abuts existing grass, cut edge of existing grass with a sharp tool or sod cutter to provide a straight line or edge of sufficient depth that allows new sod to be placed level with existing grass.

.4	Level and firm sodbed by using a water-filled roller of sufficient weight.

.5	Approximately 24 hours prior to sod placement, lightly moisten the sodbed especially during periods of high temperature and dry soil conditions.  Avoid over-watering the area.  Ensure soil has adequately dried after watering to permit sod placement without tracking.  Where necessary, lightly rake and level any disturbed soil that may occur.

# 3.3	fertilizer – initial application

.1	Apply starter type fertilizer directly to surface of approved topsoil a maximum of 48 hours prior to sod placement.

.2	Spread 12-51-0 or 16-32-6 (N-P-K) or similar synthetic granular starter fertilizer evenly at 3 kg/100 m2 using a calibrated mechanical distributor prior to laying sod.  Lightly rake and incorporate fertilizer into topsoil.

# 3.4	sod placement

.1	Obtain the Province’s approval of sodbed finish grades, final tilth, surface flatness, topsoil depth and fertilizer application before laying sod.

.2	Place first row of sod in a straight line with subsequent rows placed parallel to and tightly against each other, where possible.

.3	Lay sod in smooth even staggered rows, and closely knit together that no open joints wider than 3mm are visible.  Stagger joints between adjacent rows minimum 30 cm to avoid continuous seams and to promote more uniform growth and strength.  Do not stretch or overlap any piece of sod.

.4	Lay sod smooth and flush with adjoining turf areas and hard surfaces and the top surface of curbs.  Cut sod neatly and fit tightly to edge and line.  Do not overlap any adjacent grass with new sod.

.5	Use full rolls of sod.  Small pieces of sod or sod remnants are not allowed.  Cut and remove all small, broken, irregular, unhealthy and thin sections of sod using a sharp knife.  Install healthy new replacement sod.

.6	Protect new sod from heavy foot traffic during laying.  Provide and place wood planks or sheets as necessary to prevent damage.

.7	Install “big roll” sod using specialized equipment.  Remove any reinforcement netting used for harvesting and/or placement of the sod roll prior to laying sod.

.8	Immediately after placement of sod, thoroughly water sodded areas to prevent drying.  Obtain moisture penetration to a depth of 100 mm.

.9	After sod and soil have dried sufficiently, roll sodded areas using a roller with adequate weight to obtain a smooth uniform surface and to ensure a good bond between soil and sod.  Heavy rolling to correct irregularities in grade is not permitted.

# 3.5	LAYING SOD ON SLOPES and drainage channels

.1	On sloping terrains 3H:1V or greater place erosion control netting on topsoil for reinforcement.  Secure netting in place with wooden pegs or staples set firmly into ground at intervals of 1000 mm.  Cover mesh with thin layer of topsoil.

.2	Lay sod sections perpendicular to slope with staggered joints.  Initiate sod installation from bottom of slope and progress upward.  Securely anchor each piece or section of sod in place with 4 to 6 wooden pegs driven flush with surface of sod to prevent shifting of sod.  Lay “big roll” sod parallel to slope, with sufficient wood pegs installed to secure the sod roll in place.

.3	Drainage Channels: place sections of turfgrass sod perpendicular to direction of flow with staggered joints and adequately pegged to resist washout during the establishment period.  Lay “big roll” sod in direction of flow and adequately pegged with 6 to 9 wooden pegs per square metre to secure sod in place.

# 3.6	PROTECTION OF SODDED AREAS

.1	Protect sodded areas from all damage, trespass, disturbance, or other construction activity after sodding operations are complete.  Erect and maintain temporary barricades, guards, signage and fencing to protect sod.  Remove protection after sod is well established.

.2	Damaged sod resulting from inadequate protection shall be removed and replaced with new sod at Contractor’s expense.  All sod repairs shall be fully established prior to final acceptance.  The Province reserves the right to extend Contractor’s warranty on replacement sod if deemed necessary by the Province.

.3	Remove excess water from sodded areas.

# 3.7	MAINTENANCE and plant establishment

SPEC NOTE: edit specification item to meet requirements of landscape project.

However, delete this “Maintenance And Plant Establishment“ item if Section 32 99 00 - Exterior Landscape Maintenance is used in the contract.  This item is only for use on smaller landscape projects that involve landscape upgrades, restorations or repairs.  Section 32 99 00 (Exterior Landscape Maintenance) shall be used for all new and larger landscape projects that involve a complete new landscape development for a site.

.1	Maintenance requirements shall be performed weekly and as frequently during the week to establish and maintain grass in a healthy, vigorous growing condition.

.2	Provide clean fresh water, equipment, water tanker, methods of transportation, hoses, sprinklers and other equipment.  Apply water with a fine spray to ensure sodded areas are adequately watered.  Record quantity of water provided on site in maintenance log.

.3	Watering:  apply water regularly in sufficient quantity and frequency to maintain moist soil conditions to a depth of 75 to 100 mm to promote healthy and vigorous sod growth and ensure sodded areas become established and rooted in place.  Once sod is firmly rooted, the frequency of subsequent waterings shall be carried out when necessary to ensure healthy and vigorous growing conditions are maintained.  Monitor rainfall conditions on site prior to all scheduled waterings.

.4	Mowing:  first mowing shall not be attempted until turfgrass sod is firmly rooted and secured in place and sufficient top growth is tall enough to cut.  Not more than 30% of the grass-leaf shall be removed during the initial and subsequent mowings.  Cut and maintain sodded areas at a height of 70 to 80 mm.  Do not mow when turf is wet.  Remove excessive clippings that will smother grassed areas.

.5	Edge and trim sodded areas along hard surfaces and other fixtures to maintain a neat appearance.  Do not use line trimmers around trees and shrubs.

.6	Supply all necessary equipment, accessories and labour in use of building water sources where available.

.7	Remove all litter, leaves and other foreign debris from sodded areas.  Ensure all debris including animal waste are removed before mowing sod.  Dispose of collected debris off site at approved municipal disposal facility.

.8	Fertilizing:  fertilize sodded areas thirty (30) days after sodding.  Subsequently, provide additional applications of turf fertilizer during the growing season or maintenance period at approximately 60 day intervals.

.1	Use a complete synthetic granular slow release turf fertilizer such as 24-6-12 or 32-5-10 (N-P-K) for sodded areas and apply at approximately 3kg/100m2.  Water in well after each application.

.2	When maintenance period commences toward late summer or end of growing season postpone additional fertilizer applications (after August 15th) with a high nitrogen content until the beginning of the next growing season.

.9	Replace sodded areas showing root growth failure, deterioration, bare or thin spots and shrinkage with new sod.  Sodded areas which have been damaged by any means or cause including ruts, scalped turf, erosion, washouts, drought, diseases and by improper maintenance practices shall be restored.  All sod repairs must be completed and the sod well established prior to final acceptance.

.10	Fill minor gaps between sod sections with topsoil to prevent sod edges from drying.

.11	Control and eliminate weeds, disease, insects and other pests in sodded areas.  Use integrated pest management practices to avoid pesticide use.  Pesticides shall only be used if IPM (Integrated Pest Management) practices and techniques are not practical or efficient in managing pests and the use of pesticides is approved by the Province.

.12	Repeat rolling of sod as necessary to maintain a smooth grass surface.

.13	Spring Clean-Up: where termination of sod maintenance is not achieved in the same year as sod installation, the Contractor shall be responsible for spring clean-up of sodded areas that includes removal of dead vegetation, leaves, debris, and any sand or rock chips resulting from winter snow removal operations.  Repair damaged or deficient sodded  areas.  Complete spring clean-up by first week of May, weather permitting.

.14	Maintenance Inspections:  the Province shall conduct random site inspections to evaluate Contractor’s workmanship and performance.  Site meetings shall be scheduled to review and evaluate sodded areas and turf maintenance.  Contractor shall promptly correct all deficiencies noted by the Province.

# 3.8	maintenance log requirements

SPEC NOTE: delete this item (Maintenance Log Requirements) if Section 32 99 00 - Exterior Landscape Maintenance is used in the contract.

.1	Maintain and complete a maintenance log for each day of maintenance activity throughout maintenance period.

.2	Submit maintenance log data to the Province weekly or as directed by Province for verification and approval of services performed.  Contractor shall ensure maintenance log data is true and accurate.  Site supervisor shall complete and sign maintenance log.

.3	Maintenance Invoices:  invoices submitted without verifiable maintenance log data to support invoice shall not be accepted.  The Province reserves the right to reduce payment for unacceptable maintenance services deemed not in compliance with specifications.

.4	Record all maintenance activities including date/time of activities, quantity of water applied on site, materials used and quantity in maintenance of sodded areas, location where activities were carried out, number of employees and name of supervisor on site.

# 3.9	Final ACCEPTANCE and termination of maintenance

.1	Sod acceptance shall be based on the following criteria at end of maintenance period:

.1	Sod has been supplied, installed and maintained as specified.

.2	Sodded areas are well anchored into underlying topsoil.

.3	Sodded areas are healthy, vigorously growing, clean and free of all debris.

.4	Sod is green, even coloured, and in a viable well established condition.

.5	Sod is free of bare and dead spots, visible joints, ruts, undulations and settlement.

.6	Sodded areas are free of weeds, undesirable grass species, disease, and insects.

.7	Sodded areas have been fertilized according to fertilization program.

.2	Use specified materials to re-establish sodded lawns that do not comply with requirements for acceptance and continue maintenance of all turfgrass sodded areas until all sodded areas meet the criteria for acceptance.

.3	Upon acceptance of sod and related work by the Province at end of maintenance period, the Province will assume responsibility for the maintenance of the work.

# 3.10	CLEAN-UP and repairs

.1	During work of contract, keep all hard and soft surfaces clean and tidy.  Sweep and clean all walkways, roadways and other pavement surfaces to maintain clean appearances.  Keep catch basins, manholes, valves, irrigation components, and other hard surface features clean and free of all foreign debris.

.2	Collect all litter and other foreign debris continuously and remove from site.

.3	Remove and dispose of excess materials, litter, grass clippings and other debris to an approved disposal site.  Contractor shall be responsible for all disposal costs.

# .4	Repair and replace to original condition all damages to property resultant from the sodding operation and all damages as a result from the remedying of these defects, without additional compensation.

# END OF SECTION