Section Cover Page

Section 32 99 00 2026-04-07 Exterior Landscape Maintenance

This Master Specification Section contains:

.1	This Cover Page

.2	Specification Section Text:

1.	General

1.1 	Related Requirements

1.2	Reference Documents

1.3	Hours of Work

1.4	Measurement and Payment

1.5	Quality Assurance

1.6	Maintenance Period

1.7	Maintenance Log Requirements

1.8	Site Maintenance Inspections / Meetings

1.9	Soil Analysis

1.10	Delivery, Storage and Handling

1.11	Protection

1.12	Damage to Property

1.13	Termination of Maintenance Period

1.14	Maintenance Instructions

2.	Products

2.1	Fertilizer

2.2	Topsoil and Peat Moss

2.3	Water

2.4	Plant Protection Materials

2.5	Pest Control

2.6	Maintenance Schedule

3.	Execution

3.1	General Workmanship

3.2	Spring Clean-Up

3.3	Seed and Sod Maintenance

3.4	Tree and Shrub Maintenance

3.5	Plant Removal and Replacement

3.6	Integrated Pest Management

3.7	Pests: Weed, Insect and Disease Control

3.8	Autumn Preparation

3.9	Irrigation System Maintenance

3.10	Cleanliness of Grounds

1.	General

1.1	RELATED Requirements

.1	Seeding:	Section 32 92 19.

.2	Sodding:	Section 32 92 23.

.3	Landscape Planting:	Section 32 93 00.

.4	Irrigation System:	Section 32 80 00.

1.2	REFERENCE Documents

| .1 | “Alberta Yards & Gardens, What To Grow” published by Alberta Agriculture and Irrigation, Agdex 200/32-1. |
| --- | --- |
| .2 | “Backyard Pest Management” published by Alberta Agriculture and Irrigation, Agdex 605-2. |
| .3 | “Environmental Protection and Enhancement Act” – Alberta Environment and Protected Areas. |
| .4 | “Manual for Maintenance of Grounds” published by Alberta Infrastructure. |
| .5 | ANSI A300: pruning standards and practices of the (ISA) International Society of Arboriculture. |

1.3	HOURS OF WORK

.1	Perform maintenance services during regular working hours of 07:00 to 18:00, Monday to Friday unless directed otherwise by the Province.  Province reserves the right to adjust working hours depending on site activities and events.

.2	Obtain the Province’s approval to perform maintenance outside of regular working hours.

1.4	MEASUREMENT AND PAYMENT

SPEC NOTE: ensure measurement and payment conditions match other respective landscape sections in contract.  Delete reference to either stipulated price contracts or unit price contracts as applicable to contract.

.1	Stipulated Price (Lump Sum) Contracts:

.1	For payment purposes, twenty-five percent (25%), of the landscape contract value has been deemed to be allocated for landscape maintenance and warranty services specified in contract.

.2	Unit Price Contracts:

.1	Landscape maintenance services shall be paid at specified unit bid price.  Unit price shall include for all materials, equipment, labour and incidentals necessary to complete specified maintenance service tasks.

.3	Progress payments shall be made for work satisfactorily completed in accordance with contract specifications.  Payment shall be subject to the Province’s review and acceptance of Contractor’s workmanship and performance.

.4	The Province reserves the right to reduce monthly payments when:

.1	contractor submits false maintenance log data,

.2	contractor provides unsatisfactory, inadequate or incomplete maintenance services,

.3	inadequate or improper materials are supplied and used on site,

.4	contractor fails to perform specified maintenance services.

# 1.5	quality assurance

.1	Site Supervisor:  maintenance services shall be completed under the on-site direction and supervision of a competent, experienced and knowledgeable site supervisor.  Site supervisor shall possess a Journeyman Landscape Horticulturist certification or have other equivalent qualifications acceptable to the Province.

.2	Contractor’s staff, working on site, shall be well trained, knowledgeable and experienced in performing landscape maintenance services.

.3	Weekly Site Visits:  adequate site visits shall be provided each week with sufficient maintenance staff and equipment to ensure required maintenance tasks and services are completed in accordance with these specifications.

.4	Equipment:  provide appropriate and well-maintained equipment, tools and other accessories of sufficient type, capacity, and quantity to safely and efficiently perform all maintenance services.  Equipment shall have proper safety devices maintained at all times while in use.  If equipment is defective or unsafe and/or operated in an unsafe manner, the Province shall direct the Contractor to remove such equipment and/or the operator from site.

.5	Employee Attire:  workers shall be adequately and appropriately attired at all times while performing maintenance services.  Wearing inappropriate or offensive clothing is not allowed on site.  Reflective brightly coloured safety vests shall be worn where safety and site visibility is a concern or when directed by the Province.

.6	Regulatory Requirements:  perform work in accordance with all applicable laws, codes and regulations required by authorities having jurisdiction over such work.  Submit copies of all permits and licenses required by local authorities.

.7	Site Security:  Contractor’s personnel shall carry personal identification at all times while on site.  Present identification when requested by the Province or other site security staff.

.8	Submittals:  submit any requested information and documents requested by the Province or as indicated in contract documents for the Province’s review and approval.

.9	Safety Precautions:

.1	Contractor shall supply and ensure that all workers use appropriate Personal Protective Equipment (PPE) at all times as required by Alberta’s Occupational Health and Safety Act, Regulations and Code.  Required (PPE) shall include but is not limited to hearing protection, hard hats, safety glasses, gloves, florescent safety vests, first aid kits, spill kits and fire extinguishers.  All employees shall wear CSA approved safety shoes.

.2	Contractor shall provide training and ensure all workers practice appropriate safety measures and safe use of tools and equipment in accordance with WHMIS (Workplace Hazardous Materials Information System).

.3	The Province reserves the right to have the Contractor remove any employee from site if not wearing personal protective equipment or if not practicing safe work procedures.

1.6	MAINTENANCE PERIOD

SPEC NOTE: edit item to indicate specified length of maintenance period in contract. Ensure that the specified period matches the maintenance period specified in other respective landscape sections.

.1	Maintain exterior landscape work from commencement of work until minimum [one (1)] [two (2)] year(s) after Date of Interim Acceptance of the landscape portion of Work in Contract.

.2	Maintenance services shall generally be performed during the landscape growing season from May 1st to October 31st, weather permitting.  The Province, however, reserves the right to have maintenance services commence at an earlier date and terminate at a latter date based on existing climatic conditions, soil moisture and plant health.

.3	Any additional maintenance beyond the specified maintenance and warranty period required for Contractor to achieve final acceptance shall be provided at Contractor’s own expense.

.4	The Province reserves the right to extend maintenance period and/or reduce progress payments for maintenance services any time Contractor neglects to provide and adequate maintenance services in accordance with contract specifications.

1.7	MAINTENANCE LOG requirements

.1	Maintain a maintenance service log for each day that maintenance services or activities are performed throughout the maintenance / warranty period.

.2	Maintenance service log shall be kept at the Province’s site office (if applicable) or submitted to the Province electronically each week as directed by the Province.  Site supervisor shall complete, sign and submit maintenance log.

.3	Record all maintenance activities including date/time of activities, materials and quantity  used on site, location where activities were carried out, name of maintenance supervisor,  number of employees on site and other relevant information.

.4	Maintenance Invoice:  approval of invoices for payment is subject to submittal of verifiable maintenance service log data that supports all services being invoiced.  Services and other tasks shall have been performed and completed in accordance with contract specifications.  Maintenance services and other tasks being invoiced shall be subject to the Province’s review and approval.

1.8		SITE MAINTENANCE INSPECTIONS / MEETINGS

.1	Regular site meetings with the Contractor shall be scheduled by the Province,  to review and evaluate Contractor’s performance and workmanship.

.2	The Province shall conduct random site inspections, at their discretion, throughout the maintenance period to evaluate the site landscape and maintenance being performed.

.3	Deficient work noted by the Province shall be corrected within three days of notification from the Province or as directed otherwise.

1.9	soil analysis

.1	Perform horticultural soil tests every second year for both turf and planting locations.  Arrange and pay for services of an accredited testing laboratory.

.2	Submit soil test analysis and recommendations to the Province.

.3	Amend soil conditions in accordance with soil analysis report and indicate all corrective measures taken in maintenance service log.

1.10	DELIVERY, STORAGE AND HANDLING

.1	Remove all equipment and materials off site each day unless on site storage is approved by the Province.

.2	Store tools, equipment and materials in a secure area when not in use during period of operation and at the completion of each scheduled task.

.3	Contractor shall be present to accept delivery of all equipment and/or material shipments.  The Province will not knowingly accept, unload or store anything delivered to site for Contractor’s use.  Inadvertent acceptance of delivery by the Province shall not constitute acceptance or responsibility for any of the materials or equipment.

1.11	PROTECTION

.1	Protect plant material and turf areas against damage during the maintenance period until well established.  Provide fencing, barriers, signs and other temporary protections as necessary to ensure adequate protection is established.

1.12	DAMAGE TO PROPERTY

.1	Contractor shall be held directly responsible for all damages to the property, the personal property of all site employees, staff and visitors and the property of adjacent land-owners resulting from the actions of the contractor, the contractor’s employees, subcontractors or representatives who provide service under this contract.

.2	Contractor shall immediately report all damages to the Province.

.3	Repair or replace all damaged property to its original condition or better as directed by the Province.  Contractor shall correct all damages within timeframe set by the Province and be responsible for all costs.

.4	Scalping of turf, mechanical damage or injury to plants, improper plant pruning, inadequate plant protection, and damages resulting from improper use of chemical pesticides and fertilizers are examples of damages that shall require repair.

1.13	termination of maintenance period

.1	Work performed under this exterior landscape maintenance section shall be accepted by the Province at the end of the specified maintenance period provided all requirements for final acceptance have been satisfactorily achieved.  Requirements for acceptance are indicated in each applicable landscape section.  All warranty related work shall have been corrected during the maintenance period to ensure final acceptance.

.2	Contractor shall request a termination of landscape maintenance inspection in writing to the Province.

2.	Products

2.1	FERTILIZER

.1	Synthetic Turf Fertilizer:  apply a complete, premium grade, granular fertilizer, containing minimum 50% of total nitrogen derived from a slow or controlled release nitrogen source, with all essential macronutrients plus 1%-2% iron and other micronutrients to maintain turf in a healthy, vigorous and green condition.

.2	Do not use any "weed and feed" fertilizer.

.3	Plant Fertilizers:  apply premium grade fertilizer for plants as directed by the Province or in accordance with soil analysis (if applicable).  Use slow or controlled released fertilizers on site unless directed otherwise by the Province.

2.2	TOPSOIL and peat moss

.1	Imported Topsoil:  supply fertile natural loam, capable of sustaining healthy growth. Topsoil shall be loose and friable, free of subsoil, clay lumps, stones in excess of 20 mm, live plants, roots, wood debris, invasive and prohibited noxious plants and their reproductive parts, soil-borne pathogens, organic and inorganic materials, toxins, litter, and foreign matter.  Topsoil supplied to site shall meet the following requirements:

.1	Minimum 6% organic matter.

.2	Acidity/alkalinity shall range from 5.9 pH to 7.0 pH.

.3	Electrical Conductivity (E.C.) - level of soluble salts shall not exceed 1.5 dS/m.

.4	Texture:  “Loam Topsoil” in accordance with Canadian System of Soil Classification.  Topsoil shall fall within an allowance of 5% of the values stated in the table below:

| Soil | Sand (%) | Silt (%) | Clay (%) | Class |
| --- | --- | --- | --- | --- |
| Topsoil | 40 | 34 | 26 | Loam |

.2	Sphagnum Peatmoss:  decomposed plants, fairly elastic and homogeneous, free of decomposed colloidal residue, wood, sulphur and iron.  Minimum of 80% organic matter by mass, pH value between 4.5 and 6.0.  Furnished in an air-dry state, packed in standard bags or bales showing name of manufacturer.

2.3	water

.1	Supply clean fresh water, water tanker, hoses, pumps, sprinklers, and other accessories and equipment, and adequately and efficiently apply water to all turf and plant materials.

.2	Record quantity of water supplied and applied on site in maintenance log.

.3	The Province shall allow use of water from building sources (where available) at no cost.  Contractor shall supply all hoses and other equipment when using site water sources.

2.4	PLANT PROTECTION MATERIALS

.1	Rodent, Animal and Sun Protection:  supply the following or similar as necessary:

.1	Wire mesh:  galvanized wire mesh, electrically welded 1.4 mm wire with 12.5 x 12.5 mm mesh openings c/w fasteners.  Enclosure diameter shall be minimum three times the trunk diameter or caliper and minimum 900 mm in height.

.2	Plastic:  perforated white spiraled horticultural strip.

2.5	PEST CONTROL

.1	Chemical Pest Control:  supply and apply required chemical pesticides including herbicides, insecticides and fungicides to control or suppress pest populations when Integrated Pest Management (IPM) practices and techniques are considered ineffective.

.2	Use, handling and application of pesticides shall meet regulations of Alberta Environment’s Environmental Protection and Enhancement Act.  Submit the following information for the Province’s review and approval before applying any pesticide:

.1	Identification of specific pest(s) on site that requires control.

.2	Trade name of chemical pesticide and manufacturer’s instructions for use.

.3	Manufacturer’s material safety data sheets for each chemical pesticide.

.4	Name and credentials of licensed pesticide applicator(s).

.5	Copy of the license obtained for pesticide application showing the authorization number and expiry date, and a copy of the service approval received from Alberta Environment.

.3	Pesticide Application Records:  pesticide application record books shall be completed by the certified licensed applicator in accordance with Alberta Environmental Protection and Enhancement Act.  Submit record books at completion of each pesticide application. Provide information regarding target weed, insect or other pest, mode, type, and rates of application and results, including date, time, weather conditions and name of applicator.

# 2.6	MAINTENANCE SCHEDULE

.1	Submit a legible and detailed maintenance schedule at commencement of maintenance services each year.  List all daily, weekly, and monthly maintenance services and other tasks to be performed.  Review schedule with the Province for approval.

3.	Execution

3.1	GENERAL WORKMANSHIP

.1	The Province shall be the sole assessor of Contractor’s performance and workmanship.

.2	Schedule timing of operations to growth, weather conditions and use of site.  Do each operation continuously and complete within reasonable time period.

.3	Do not perform work in any location or manner that may endanger the health and safety of the public.

.4	Collect and dispose of excess material and debris to approved municipal disposal site following collection.  Contractor shall be responsible for all disposal costs.

.5	Coordinate maintenance practices with the Province.  Alter maintenance schedules, when necessary, to accommodate the Province’s site activities, events and functions.

.6	Contact the Province when specified maintenance services cannot be met for any reason.

.7	Submit name and contact number of company representative(s) for immediate or emergency service when necessary.

.8	Contractor shall ensure copy of landscape contract specifications remain on site for employee use and reference at all times.

.9	Contractor’s principal owner and/or site supervisor shall attend all site meetings called by the Province to review workmanship and performance.

.10	Promptly correct all maintenance deficiencies noted by the Province.  Deficient maintenance work or services shall be corrected within three days of the Province’s notification or as directed otherwise by the Province.

3.2	SPRING CLEAN-UP

.1	Complete spring clean-up by May 15th, weather permitting.  When completion date cannot be met, promptly advise the Province of the circumstances.

.2	Sweep, collect and remove sand, rock chips, salt debris and other waste and litter that has accumulated during winter months from all turf and planted areas throughout the site.  Dispose of all collected debris to approved municipal disposal site.  Contractor shall pay all disposal costs.

.3	Collect and remove from site all dead vegetation, leaves, and litter from turf areas.  Rake clean dead patches of grass and fungal growth in lawn areas resulting from snow mold, (where applicable).  Repair turf damages with topsoil and grass seed to match existing.

.4	Remove and dispose of protective coverings and mulch used for winter protection, (where applicable).

.5	Sweep and clean all walkways, stairs and other hard surface areas.  Sweep and remove accumulated debris along all road gutters to maintain clean site appearances.

.6	Remove and store on site all stakes, sand/rock chip storage containers, snow fencing, and other materials supplied by the Province.  Contractor supplied materials including winter protection materials shall be removed off site.

.7	Clean all plant beds, tree wells, tree grates, raised planters, hedges, hard surface mulched beds and all other landscaped areas of plant debris, rock chips, leaves, litter, and other undesirable foreign matter.  Rake and clean by hand plant beds containing shredded wood mulch.  Remove all collected debris from site to approved municipal disposal site.

.8	Loosen and lightly cultivate non-mulched planting soil without disturbing roots of permanent plantings.  Re-install loose plant bed edging materials and/or replace if deteriorated or damaged.

.9	Roll turf areas lightly where grass has lifted due to frost action.  Repair damaged or deficient turf areas.

3.3	seed and sod MAINTENANCE

.1	Watering – Turf Areas:

.1	Provide adequate watering services to ensure the proper establishment of all turf areas and to prevent turf and underlying soil from drying out.  Apply water during early morning or evening to achieve efficient use of water.  Ensure healthy vigorous growing conditions are maintained.

.2	Seeded Areas: apply water frequently to supplement rainfall.  Ensure continuous 20 mm moisture penetration from time of seeding to 90% germination.  From 90% germination to full turf establishment apply water to supplement rainfall and to ensure 25 mm of water each week.  Supply and operate portable and mobile irrigation systems for watering as necessary until grass is well established.

.3	Sodded Areas: apply frequent and sufficient water from time of sodding.  Ensure moisture penetration of 75 to 100 mm to keep new sod and underlying soil from drying out and to maintain healthy vigorous growing conditions.  Continue to apply water until sodded areas are deemed to be well established and anchored into the underlying soil.

.4	Irrigated Turf Areas: where underground irrigation systems exists on site, Contractor shall coordinate operation of irrigation system with the Province to ensure irrigated turf areas receive adequate water.

.5	Water Supply: Contractor shall supply clean water, water truck, pumps, portable irrigation equipment and labour to operate equipment and apply water to all non-irrigated sod and seeded areas ensuring healthy turf conditions are maintained.

.6	Building Source Of Water: supply hoses, sprinkler equipment, and other accessories when using any available building water source that the Province makes available for use at no cost.

.7	During periods of drought stress, provide additional watering services to all sod and seeded areas to maintain healthy turf conditions.  Never allow turf areas to dry out, deteriorate and perish.

.2	Topdressing and Reseeding:

.1	Topdress and reseed all seeded areas which show root growth failure, deterioration, bare, burnt and thin spots, ruts, wash-outs and/or erosion or which have been damaged by any means or cause, including replacement operations.  Seedbed repairs shall be ongoing and continuous each month.

.2	Mow grass to height of 40 mm.  After mowing, rake thoroughly, removing loose and dead grass, stones and debris.

.3	Spread topsoil to maximum thickness of 15 mm, filling in low areas and bare spots.  Place sufficient topsoil on severely damaged turf areas and rake level with adjacent finish grade.

.4	Overseed turf areas with a quality seed mixture equivalent to existing grasses and approved by the Province.  Use a broadcast spreader and seed heavily at minimum rate of 4 kg/100 m2.

.5	Lightly rake seed into topsoil.  Roll lightly as necessary.  Apply appropriate fertilizer to all seed repairs.

.6	Maintain seed repairs using all means necessary including water to ensure healthy and vigorous growth.

.3	Sod Replacement:

.1	Replace and repair all areas of dead or unhealthy sod or which has been damaged by any means or cause.  Neatly cut and remove square or rectangular areas of deficient sod and replace with new healthy sod.  All sod repairs shall neatly and tightly blend into existing undisturbed turf areas.

.2	Rake existing topsoil before installing new sod.  Add topsoil to fill uneven and low areas.

.3	Butt new sod tightly to adjacent existing sod.  Finish grade of new sod shall match adjacent undisturbed turf.  Place topsoil to fill open and exposed joints.

.4	Water to ensure penetration of 80 mm and at frequent intervals to maintain healthy vigorous sod growth.

.4	Turf Establishment and Erosion:

.1	Until turf areas are established, the Contractor shall be responsible for replacing soils that have eroded into hard surface areas.  Residual soils on hard surfaces shall be removed and if not mingled with objectionable materials may be re-used in eroded areas.  In the event this is a continuous problem, the Contractor shall install “jute mat” or other methods to prevent the erosion problem until the seeded turf areas are established.

.5	Fertilizing:

.1	Apply a custom mixed granular turf fertilizer as specified.  Use mechanical equipment to spread fertilizer.  Check calibration of spreader to ensure specified application rate is used.

.2	Spread 50% of fertilizer in one direction, then 50% at right angles.

.3	Synthetic Fertilizer:  apply the following slow release granular fertilizer unless approved otherwise by the Province or in accordance with soil analysis report.  Indicate total quantity of fertilizer applied on site in Maintenance Log.

.1	Spring Application:  all turf areas.  Apply 26-14-6 or similar slow release granular fertilizer at rate of 3 kg/100 m2 by May 20th.

.2	Summer Application:  all turf areas.  Apply 24-5-11 or similar slow release granular fertilizer at rate of 3 kg/100 m2 by July 15th.

.3	Fall Application: all turf areas.  Apply 12-3-18 or similar slow release granular fertilizer at rate of 2 kg/100 m2 by September 30th.

.4	Water:  apply water immediately after spreading fertilizer.  Supply water, trucking and accessories and apply required water to all turf areas.  Use site irrigation system (if applicable).  Alternatively, Contractor may time fertilizer applications prior to natural rainfall that will activate the fertilizer and produce the desired response.  Ensure moisture penetration of 50 mm minimum.

.5	Advise the Province prior to any turf fertilizer application.  At completion of service, provide documentation to support type and quantity of turf fertilizer applied on site.

.6	Mowing - Maintained Areas:

SPEC NOTE:  Edit item to suit turf (sod, seed or native grass seed) installed in contract.

.1	Mowing Height:  cut and maintain turf areas at required heights respective of the growing season.  During periods of active growth, turf shall be mowed at minimum required mowing height.  During hot dry conditions or slow growth periods turf shall be mowed at increased or maximum heights to maintain turf health.  Turf shall generally be cut and maintained as follows:

.1	Sodded Turf:  cut and maintain turf between 60 mm to 80 mm in height.

.2	Seeded Turf:  during seed establishment, provide weed control in newly seeded areas by mowing and maintaining weed growth to 100 mm height.  Remove all weed and other plant debris from seeded turf areas.  Provide initial cut when seeded grass growth is approximately 100 mm in height over 75% of the seeded area.  As the seeded turf becomes established mow at a height of 80 mm to 90 mm.

.3	Non-Maintained Native Seed:  continuously remove weed growth during seed establishment by physical means only.  Collect and dispose of weed debris off-site.  Any mowing that may be necessary shall be limited to once per season in late spring at a minimum height of 150 mm and only if requested by the Province.  Ensure no wildlife are nesting before mowing.  Remove excessive accumulations of clippings and dispose off site.

.2	Mowing Schedule: mow sodded turf areas on weekly intervals during periods of active growth.  During periods of hot dry seasonal climatic conditions or when turf growth has slowed mow turf less frequently or when directed by the Province at an increased height to maintain turf health.  Seeded turf areas shall only be mowed as the need occurs during the seed establishment period and weekly once the turf is well established.

.3	Turf mowing shall generally occur when one-third of grass blade can be removed during a single cutting to achieve required turf height.  Mowing turf when insufficient growth is evident or cutting turf lower than specified height shall be considered unacceptable.

.4	Mow turf areas only when dry and in one continuous operation.  Keep mower blades sharp to cut turf clean and even.  Turf mowed when wet or with dull blades that tears and leaves ragged leaf edges is unacceptable.

.5	During periods of extended moisture and excessive turf growth, increase mower blade height and pre-cut turf at an increased height.  Remove clippings.  After turf dries, lower blade height to original specified height and double cut turf a second time to achieve required height.

.6	Remove clippings after each turf mowing.  However, small and unnoticeable clippings may be left on lawns that are regularly mowed at the desired height.

.7	Prior to each mowing remove all litter, debris, rocks, and animal waste from all turf areas and adjacent landscaped grounds.

.8	Change direction of cut with each mowing to avoid soil compaction and turf wear or ruts from mower wheels.  In the event that turf damage or ruts do result,  immediately repair the damaged turf.

.9	During each cutting operation, temporarily relocate movable site furnishings (if applicable).  Replace to approximate original location after mowing is completed.

.10	Edging: use a mechanical power edger with fixed steel blade to cut and provide neat, uniform, and cleanly defined edges along all sidewalks, concrete curbs, and other hard surfaces on site.  Do not use monofilament trimmers for edging.  Provide monthly edging services from June to September.  Remove all edging debris from site.

.11	Trimming:  trim grass areas as follows:

.1	trim grass along fences, walls, sign posts, structures, monuments and adjacent to fixed objects using a mechanical trimmer.  Trim turf at a height no lower than 70 mm.  Never scalp turf or damage any plant when using trimmers.  Remove resulting plant debris from site.

.2	use hand trimmers to trim grass adjacent to trees and other plants to prevent damage to tree trunks, plant stems and roots.  Do not use string line trimmers near live plants.

.3	cut and neatly trim grass by hand around irrigation sprinkler heads (if applicable) to ensure proper water coverage.  Trim grass around valve boxes, manholes, valves and other surface features in turf areas.

.12	Clean all sidewalks, stairs, roads, parking lots, road gutters, building structures and other hard surface locations of all grass clippings after each mowing.

.7	Turf Aeration:  Provide aeration services every year as necessary.

.1	Identify all sprinkler heads and other obstructions with flags before aerating.

.2	Use a vertical motion coring aerator with hollow tines that penetrate the lawn, extract and deposit soil cores.  Aeration cores shall be of minimum 75 mm depth and spaced minimum 90 mm apart.  Ensure soil has adequate moisture before aerating.

.3	Aerate turf areas in minimum two different directions to obtain good coverage.  Remaining soil cores may be left on site.  Perform aeration in early spring or in early fall.

.8	Turf Dethatching:  Service to be provided every second year during maintenance period.

.1	Identify all sprinkler heads and other obstructions with flags before dethatching.

.2	Use a vertical cutting mower or adjustable power rake to dethatch turf areas in early spring when dry.

.3	Cross lawn areas two to three times in varying cross directions.  Collect resulting thatch debris and remove from site.

3.4	TREE AND SHRUB MAINTENANCE

.1	Monitoring:

.1	Regularly monitor and visually inspect the health and care of all plants on site to ensure proper plant care is provided and that all pests are properly identified in accordance with specifications.

.2	Indicate results of each plant monitoring inspection in maintenance log.

.2	Trees and Plant Beds:

.1	Provide proper and adequate maintenance to all plants, plant beds and mulched areas each week.  Ensure healthy and vigorous growing conditions and aesthetically clean and pleasing site appearances are provided and maintained.

.2	Cultivate non-mulched (if applicable) plant beds and tree wells in upper 40 mm of soil to maintain a loose friable soil free of perennial weeds and grass, including their roots.  Pull weeds and roots by hand.  Do not damage any tree roots or mound soil around base of tree trunk following soil cultivation.

.3	Collect and remove all litter, plant debris, rubbish, animal waste, foreign plant growth and other debris each week.  Dispose of collected materials off-site to approved municipal disposal facility.  Remove all exposed rocks larger than 12 mm in diameter that may appear in planted areas and dispose off-site.

.4	Weeding: remove all perennial grasses and weeds from all plant beds, tree wells and within shredded wood mulch and rock beds each week by hand.  Do not use string line trimmers near live plants.  If hand-weeding or other methods to control weeds and other foreign plant growth is not feasible supply and use contact spray herbicides but only if damage to plant growth will not occur.  Apply herbicides in accordance with manufacturer’s instructions and with the Province’s approval.  Verify that herbicide is appropriate for use with existing plant materials.  Contractor shall be responsible for any damages.

.5	Edge and maintain a neat, clean and circular tree well each month.  Keep tree well area free of weed and perennial grass growth.  Remove all resulting edging and weeding debris from site.  Do not damage or cut roots of plants.

.6	Edge along all plant beds and hedges evenly to depth of 100 mm to maintain original line and shape.  Use a sharp spade to make clean and neat angled cuts each month.  Where plants have outgrown the original edge of plant bed enlarge the plant bed by edging and removing all turf and other debris to outside edge of new plant growth.  Remove all resulting debris from site.  Install additional mulch as necessary.

.7	Tree Grates: (as applicable), remove tree grates on a monthly to bi-monthly basis and clean the planted well area.  Remove all weeds, debris and other litter.  Replace grates after completion.  When necessary, cut and resize tree grates to prevent girdling and other damage to tree trunks.  Maintain a minimum 50 mm spacing between plant tissue and tree grate metal.  Protect any cut metal surface by re-finishing in accordance with manufacturer’s instructions.

.8	Place planting media to correct any settlement that occurs in planted areas and to correct other grading deficiencies.

.9	Mulch: rake, level and re-spread shredded wood mulch to achieve a fresh appearance and to correct any disturbances each month.  Clean and remove all debris, litter, perennial weeds, grass and other foreign matter from all mulched areas weekly.  Replace deteriorated shredded wood mulch each year with similar matching wood mulch and original depth.

.10	Straighten all leaning or sagging plants.  Replant if necessary.

.11	Maintain soil lips around outer edge of tree pit including shredded wood mulch within the tree pit.  After completion of warranty period level soil lips or remove excess soil from site.  Replace shredded wood mulch yearly.

.3	Watering of Plants:

.1	Monitoring:  regularly test and monitor soil moisture conditions and a plant’s need for water to ensure adequate health and survival.  Natural rainfall should be considered in determining amount and frequency of watering.  Regularly review plant watering needs with the Province.  Contractor, shall be responsible for ensuring that sufficient moisture is provided each week, as necessary, to maintain an adequate level of moisture within the root systems and ensure healthy plant growth and survival.

.2	Use moisture sensor devices to test and measure availability of moisture in plant soil areas throughout site.  Record data in maintenance log.

.3	Water plants regularly each week as necessary to maintain healthy and vigorous growing conditions and to ensure survival.  During periods of warm weather and dry soil conditions provide additional waterings to maintain plant health.

.4	Plant Watering:  apply a minimum 40 liters of water per 25mm of tree trunk diameter (measure at 300mm above ground level) when watering.  Apply sufficient water during each application to obtain moisture saturation of each plant’s root ball.  Record total quantity of water applied in maintenance log after each site visit to perform plant watering services.

.5	Supply clean fresh water, equipment, methods of transportation, water tanker, hoses and sprinklers, attachments and other accessories, including labour to adequately and efficiently apply water to all plants where other means of watering are inadequate, inefficient or not available.

.4	Fertilization Requirements in Early Spring:

.1	Apply plant fertilizer as directed by the Province or in accordance with soil analysis.  Use controlled or slow release plant fertilizers only.  Record all  fertilization activities in maintenance log.

.2	Trees: apply 10-6-4 or similar plant fertilizer to all trees by June 1st each spring at rate of 40 g/25mm of tree caliper.  Use a deep root feeder in application of plant fertilizer and water to ensure each plant is thoroughly fertilized and watered.

.3	Alternatively, place 16-10-9 or similar fertilizer spikes in ground at rate of one spike per 25 mm of caliper per tree around dripline of tree.

.4	Plant Beds:  spread a slow release and balanced granular fertilizer such as 14-14-14 by June 1st each spring at rate recommended by manufacturer into upper surface of all planting beds.  Lightly rake into soil (if applicable) and apply water.  However, where plant beds are covered in shredded mulch apply a similar liquid fertilizer using specific equipment to apply.  Record in maintenance log.

.5	Supply clean water, water hauling equipment and all other accessories as necessary to apply adequate water after fertilizing to ensure penetration of fertilizer into soil and roots.

.5	Soil Conditioning:

.1	Maintain specified soil conditions in plant beds to promote optimum growth and health for each plant.

.2	Supply and spread soil amendments in accordance with soil analysis.

.6	Staking and Tree Protection:

.1	Keep stakes and guy wires taut and plants plumb.  Do not allow trees to be girdled from improper installation and maintenance of guy wires.

.2	Remove support stakes and staking accessories one year after installation or when directed by the Province.

.3	Install and keep plant protection materials in proper repair and adjustment as required.

.6	Pruning:

.1	Prune in accordance to ANSI A300 standard practices for tree care operations.  The Province will be the sole assessor of the quality of all pruning operations.  Correct all deficient pruning work as directed by the Province.

.2	Prune to maintain the natural shape, form, character and habit of the plant.  Pruning shall be performed in a manner that encourages healthy natural growth patterns.  Do not clip or shear shrubs into balled or boxed forms as this type of pruning shall be deemed improper and unacceptable by the Province.

.3	Make all cuts above the bark ridge and branch collar to leave the ridge and collar intact for healing.  Pruning of new plantings shall be performed by Certified Arborists or by a trained and certified Landscape Horticulturist.

.4	Use sharp pruning tools and other equipment to prune plants without tearing and ripping plant tissue.  Sterilize pruning tools after completion of each plant cutting operation and especially after pruning diseased plants.

.5	Do not shear or top any plant, strip or clear all lower branches of trees.  Avoid raising of the crown to provide vertical clearance on newly planted trees.

.6	Do not leave undesirable stub cuts when pruning.

.7	Limit pruning to removal of dead, diseased or injured branches, stray branches, double leaders, watersprouts, suckers and to compensate for loss of roots as a result of transplanting.  Remove undesirable, rubbing and crowded limbs and maintain an adequate clearance.

.8	Remove all pruned wood, cuttings, diseased limbs and branches, debris and excess material off site to approved disposal location.

.9	Contractor shall replace all improperly pruned plants at their cost that are deemed unacceptable by the Province.

3.5	plant removal and replacement

.1	Immediately remove all dead, unhealthy and unappealing plants from site or when directed by the Province and replace with healthy new plant material.  Warranty replacement plants shall meet specifications. The Province shall be "Sole Judge" in determining what plants require removal and replacement.

.2	Replacement plantings shall be continuous during maintenance/warranty period and shall be performed in a timely manner as soon as conditions permit.  The Province shall be kept informed of any delay in obtaining or installing any replacement plants.

.3	Replacement plants must be fully established before maintenance period is terminated in contract.  Contractor shall continue to provide specified maintenance for replacement plants not fully established until deemed acceptable by the Province.

.4	Plant warranty shall be extended on all plants not fully established by end of warranty period.  The Province shall determine the extent of additional plant warranty that may be required if deemed necessary by the Province.

.5	Required replacement plantings during the maintenance period resulting from theft and vandalism shall be the the Province’s responsibility.

3.6	INTEGRATED PEST MANAGEMENT

.1 	Integrated Pest Management (IPM):

.1	Manage and control pests using IPM principles that utilizes regular monitoring to identify pests, considers various control options (biological, physical, cultural, mechanical and chemical) before implementing an effective, economical and environmentally acceptable solution to prevent and suppress pests.

.2	Implement an effective IPM program to manage weeds and other pests using a combination of methods that reduces or eliminates a reliance on chemical pesticides.

3.7	PESTS: WEED, INSECT AND DISEASE CONTROL

.1	Pest Monitoring:

.1	Pest Monitoring:  regularly monitor and visually inspect all landscape areas to identify potential pest problems and determine appropriate pest controls.  Pest problems include insect, disease and weed infestations that pose a serious and on-going threat to plant life on site.

.2	Ensure proper, positive identification of pest infestations.  Review all pest concerns with the Province.

.3	Provide a summary of pests identified and provide a description of controls implemented in the maintenance service log.

.2	General Considerations:

.1	Use of chemical pesticides are restricted as the Province supports the use of Integrated Pest Management (IPM) practices.  Advise the Province on whether IPM practices and techniques are practical or effective in managing and controlling any existing pests on site or if chemical pesticides are necessary.

.2	Determine susceptibility of plant species to pesticide damage before requesting chemical pesticides.  Request the Province’s approval before use, where chemical pesticides are deemed necessary in the management and control of pest infestations.

.3	Perform applications of pesticides in accordance with Alberta Environment’s current legislation.  Provide the Province with three days advance notification of intent to apply chemical pesticides on site.

.4	Use equipment and containers free of harmful residues not related to specific control measures applicable to situation.

.5	Certified Applicator:  when pesticides are deemed necessary to control pests, the application of each pesticide on site shall be performed by a certified pesticide applicator.  Personnel assisting the certified applicator on site shall be thoroughly trained and knowledgeable in pesticide applications and use of all equipment in accordance with Alberta Environment’s Code of Practice for Pesticides.  Maintain pesticide application record books and submit at completion of each pesticide application.

.6	Prepare and apply pesticide according to manufacturer's specifications.  Minimize drift at all times.  Erect signs to notify building occupants and the public regarding pesticide use on site.

.7	Timing:  apply pesticides at times, which limit any possibility of contamination from climatic and other factors.  Monitor weather conditions to avoid making application prior to inclement weather to eliminate potential runoff from treated areas.  All pesticide applications shall be performed outside of regular site operation hours to avoid contamination from drift and its effect on surroundings, occupants of nearby buildings and site users.

.8	Improper applications of pesticide shall be immediately terminated and corrected.  Perform additional pesticide applications two weeks after any initial application is noted as being visibly inadequate or deficient by the Province.

.3	Pest Control:

.1	Weeds:

.1	Provide continuous weed control and eradication methods throughout the site during active growth and establishment.  Ensure weed seed heads are removed before maturity.  Implement IPM practices and techniques that include cultivation and physical methods where practical before applying chemical pesticides.  Never use string line trimmers near live plants to remove weeds.

.2	Noxious Weeds: completely eliminate and remove from site all noxious weeds in accordance with government regulations.

.3	Implement thorough, effective, and continuous controls to control and eliminate weed growth within all hard landscaped surfaces.

.2	Insects and Disease:  apply pesticides based on development stage of insects' life cycles to prevent loss or damage to plant material.  Monitor turf areas and plants and apply approved pesticides to control pest infestations if IPM practices and techniques are not effective.

.3	Monitor effectiveness of each pesticide application and promptly correct any inadequate or deficient application.

.4	Repair and pay for damage caused by application of herbicides.

.5	Do not use soil sterilants.

.6	Eliminate rodents using controls and methods approved by the Province.

3.8	AUTUMN PREPARATION

.1	Leaf Removal:  remove fallen leaves each week using rakes, vacuums and other equipment to maintain clean site appearances.  Remove all collected plant debris to an approved waste recycling depot.  Continue service until leaves cease to fall.

.2	Clean all plant beds, planters, tree wells, mulched areas, and other landscaped areas weekly of all plant debris, litter, leaves, and other foreign debris.  Remove all collected plant and other organic matter to approved waste recycling depot.

.3	Keep pedestrian walkways and other hard surfaces clean of all debris on a continuous basis as the need occurs.  Ensure catch basins and all road gutters are clean and free of all debris.  Remove all collected debris and litter to approved waste recycling depot.

.4	Remove all annuals (if applicable) within one week after first killing frost and dispose at approved waste recycling depot.  Deep cultivate annual plant beds and planters (if applicable) following plant removal.

.5	Cut back foliage of perennial plants within one week of freezing weather conditions.  However, ornamental grass plants shall remain intact during winter and trimmed back the following spring.  Stake locations of cut perennials and apply organic mulch around plants for winter protection as necessary.  Thoroughly water all plants in preparation for winter.

.6	Autumn Deep Root Watering: deep root water all plants between October 1st and 15th in preparation for winter.  Supply water, trucking and other necessary accessories to adequately water each plant.  Trees to receive minimum 40 liters of water per 25 mm of trunk diameter (measure at 300 mm above ground level).  Apply water throughout tree dripline area.  Saturate root area of shrubs, perennials and other plants.  Record total quantity of water applied and location of plant watering activities in maintenance log after each day of watering activities.  Provide additional deep root watering services in late fall when warm dry temperatures are experienced to maintain plant warranty.

.7	Install appropriate protective materials to protect plants from rodent, animal and sun damages.

.8	Supply and erect snow fencing, if necessary, to protect new landscape installations from damage (if applicable).

3.9	IRRIGATION SYSTEM MAINTENANCE

SPEC NOTE:  delete this entire item if no irrigation system has been installed in contract.

.1	Maintenance of site irrigation system installed in contract shall be performed by the Contractor’s irrigation sub-contractor in accordance with requirements of the irrigation contract specifications.

.2	Contractor shall co-operate and coordinate maintenance requirements with the irrigation contractor and the Province to ensure irrigation system is set, adjusted and maintained to suit the needs and requirements of the site landscape.

.3	Contractor shall perform regular visual inspections of the irrigation system while in performance of grounds maintenance services and promptly inform the irrigation contractor and the Province of any deficient or defective irrigation equipment or operation on site.

.4	Contractor’s irrigation sub-contractor shall complete a maintenance service log detailing all maintenance activities and repairs for each day that services are provided.  Log shall be submitted to the Province following completion of services.

3.10	CLEANLINESS OF GROUNDS

.1	Site grounds shall be kept in a clean and tidy condition.  Clean-up services shall be performed each week to ensure aesthetically pleasing and desirable site appearances are continually maintained.  Sweep, clean, collect and remove all debris, litter, rubbish and pests on an ongoing basis in maintenance of site grounds.

.2	Provide prompt service within two (2) hours when directed by the Province to correct or complete clean-up services deemed inadequate or incomplete.

.3	Dispose of all collected litter and other debris off site to a municipal disposal facility.  Contractor shall pay all off-site disposal costs.  Do not dispose of any debris in the Province’s disposal bins on site.

.4	Empty all trash receptacles (installed in contract) each week or more often when necessary based on usage.  Never allow receptacles to spill-over with garbage debris.  Supply and replace heavy duty refuse bags after each trash removal and clean-up.

.5	Sweep and clean all building entrances/exits, stairs, site walkways and municipal sidewalks, and all other hard surfaces for pedestrian use each week as required to maintain clean site appearances.  Keep curbs and gutters including catch basins clean of accumulated debris and other litter.   Collect and remove all debris from site.

.6	Hand scrap and/or pressure wash building entrance areas and other locations to remove gum, soil and foreign matter when necessary.

.7	Keep site furnishings (installed in contract) clean and safe for use.  Clean cigarette snuffers and other ashtray furnishings, (if applicable).

.8	Clean washed rock, gravel, wood and other mulched surfaces of all debris, litter, weeds and foreign plant growth as the need occurs.

.9	Make weekly inspections for vandalism and damage.  Immediately report vandalism and damage to the Province.  Note all findings in maintenance log.

.10	Record all cleaning services performed each week in maintenance service log.

END OF SECTION